#include <Servo.h>

#define flameLeft A0

#define flameFront A1

#define flameRight A5

// Motor Pinleri

const int motorsag_en  = 3;

const int motorsol_en  = 5;

const int in_1  = 8;

const int in_2  = 9;

const int in_3  = 10;

const int in_4  = 11;

char bt;

int pump = 7;

int servo = 6;

// 🔴 ACIL STOP BUTONU (D2)

const int emergencyIn  = 2;  // Butonun bağlı olduğu pin

bool isDone = false;

volatile bool emergencyStop = false;

// Manuel Mod Takibi

bool manuelModAktif = false;

Servo myServo;

// ================================================================

// 🔥 AKILLI BEKLEME

// ================================================================

bool akilliBekle(int sure) {

unsigned long baslangic = millis();

while (millis() - baslangic < sure) {

// 1. Donanım (Buton) kontrolü

if (emergencyStop) {

herSeyiKapat(); return true;

}

// 2. Yazılım (Bluetooth Z tuşu) kontrolü

if (Serial.available()) {

char c = Serial.peek();

if (c == 'Z') return true;

}

}

return false;

}

// 🔥 HER ŞEYİ KAPATAN FONKSİYON

void herSeyiKapat() {

digitalWrite(pump, LOW);

digitalWrite(in_1, LOW); digitalWrite(in_2, LOW);

digitalWrite(in_3, LOW); digitalWrite(in_4, LOW);

analogWrite(motorsag_en, 0); analogWrite(motorsol_en, 0);

}

// -------- TEMEL MOTOR FONKSİYONLARI --------

void ileri(){

digitalWrite(in_1, HIGH); digitalWrite(in_2, LOW);

digitalWrite(in_3, LOW);  digitalWrite(in_4, HIGH);

analogWrite(motorsag_en, 254); analogWrite(motorsol_en, 254);

}

void geri(){

digitalWrite(in_1, LOW);  digitalWrite(in_2, HIGH);

digitalWrite(in_3, HIGH); digitalWrite(in_4, LOW);

analogWrite(motorsag_en, 254); analogWrite(motorsol_en, 254);

}

void dur(){

digitalWrite(in_1, LOW); digitalWrite(in_2, LOW);

digitalWrite(in_3, LOW); digitalWrite(in_4, LOW);

analogWrite(motorsag_en, 0); analogWrite(motorsol_en, 0);

}

void sag(){

digitalWrite(in_1, LOW);  digitalWrite(in_2, LOW);

digitalWrite(in_3, LOW);  digitalWrite(in_4, HIGH);

analogWrite(motorsag_en, 254); analogWrite(motorsol_en, 254);

}

void sol(){

digitalWrite(in_1, HIGH); digitalWrite(in_2, LOW);

digitalWrite(in_3, LOW);  digitalWrite(in_4, LOW);

analogWrite(motorsag_en, 254); analogWrite(motorsol_en, 254);

}

// -------- ÖZEL HAREKETLER --------

void sagcapraz(){

if(emergencyStop) return;

ileri();

if(akilliBekle(100)) return;

sag();

if(akilliBekle(200)) return;

}

void solcapraz(){

if(emergencyStop) return;

ileri();

if(akilliBekle(100)) return;

sol();

if(akilliBekle(200)) return;

}

void front(){

if(emergencyStop) return;

ileri();

if(akilliBekle(200)) return;

}

// -------- SERVO HAREKETLERİ --------

void servo_sag(){

for(int angle = 90; angle >= 30; angle -= 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

for(int angle = 30; angle <= 90; angle += 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

}

void servo_sol(){

for(int angle = 90; angle <= 150; angle += 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

for(int angle = 150; angle >= 90; angle -= 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

}

void servo_front(){

for(int angle = 40; angle <= 130; angle += 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

for(int angle = 130; angle >= 40; angle -= 3){

if(emergencyStop) { herSeyiKapat(); return; }

myServo.write(angle);

if(akilliBekle(100)) return;

}

}

// -------- BUTON KESMESİ --------

// Butona basıldığında (FALLING) burası çalışır

void acilDurdurmaKesmesi(){

emergencyStop = true;

herSeyiKapat();

}

// -------- SETUP --------

void setup(){

Serial.begin(9600);

pinMode(flameLeft, INPUT); pinMode(flameFront, INPUT); pinMode(flameRight, INPUT);

pinMode(pump, OUTPUT);

pinMode(in_1, OUTPUT); pinMode(in_2, OUTPUT);

pinMode(in_3, OUTPUT); pinMode(in_4, OUTPUT);

// ⚠️ ÖNEMLİ: INPUT_PULLUP modu kullanıyoruz.

// Butonun diğer ucu GND'ye bağlı olmalı.

pinMode(emergencyIn, INPUT_PULLUP);

myServo.attach(servo);

// Butona basınca pin LOW'a düşeceği için FALLING (Düşen Kenar) kullanıyoruz

attachInterrupt(digitalPinToInterrupt(emergencyIn), acilDurdurmaKesmesi, FALLING);

}

// -------- LOOP --------

void loop(){

// 🔴 1. ACİL STOP (KİLİTLİ DURUM)

if(emergencyStop){

herSeyiKapat();

// Kilidi açmak için Bluetooth bekle

if(Serial.available()){

char c = Serial.read();

// F, B, L, R, S tuşlarından biri gelirse kilidi aç ve devam et

if(c=='F' || c=='B' || c=='L' || c=='R' || c=='S'){

emergencyStop = false;

manuelModAktif = false; // Resetlenince otomatik başla

isDone = false;

}

// Eğer komut hareket komutuysa ve kilit açıldıysa hemen uygula

if(!emergencyStop){

if(c=='F') { ileri(); manuelModAktif = true; }

else if(c=='B') { geri(); manuelModAktif = true; }

else if(c=='L') { sol(); manuelModAktif = true; }

else if(c=='R') { sag(); manuelModAktif = true; }

else if(c=='S') { dur(); manuelModAktif = false; }

}

}

}

// 🟢 2. NORMAL ÇALIŞMA

else {

// --- BLUETOOTH OKUMA ---

if(Serial.available()){

bt = Serial.read();

if(bt == 'Z'){

emergencyStop = true; // Yazılımsal stop

herSeyiKapat();

return;

}

// Manuel hareket komutları

else if(bt=='F') { ileri(); manuelModAktif = true; }

else if(bt=='B') { geri(); manuelModAktif = true; }

else if(bt=='L') { sol(); manuelModAktif = true; }

else if(bt=='R') { sag(); manuelModAktif = true; }

else if(bt=='S') { dur(); manuelModAktif = false; }

}

// --- MANUEL MODDA İSEK YANGINA BAKMA ---

if(manuelModAktif) {

return;

}

// --- OTOMATİK YANGIN SÖNDÜRME MODU ---

int L = analogRead(flameLeft);

int F = analogRead(flameFront);

int R = analogRead(flameRight);

bool flameDetected = (L < 80 || F < 50 || R < 50);

if(flameDetected && !isDone){

digitalWrite(pump, HIGH);

if(R < 50){

sagcapraz();

if(!emergencyStop) { dur(); servo_sag(); }

}

else if(L < 80){

solcapraz();

if(!emergencyStop) { dur(); servo_sol(); }

}

else if(F < 50){

front();

if(!emergencyStop) { dur(); servo_front(); }

}

herSeyiKapat();

if(!emergencyStop) isDone = true;

}

else {

herSeyiKapat();

if(!flameDetected) isDone = false;

}

}

}